NetSDK Programming Manual

VERSION 1.4.0.0 (Build 090926)

2009-01-10

All rights reserved

Foreword

Thank you for using our VANGUARD devices. We are going to provide best service for you. This manual may contain spelling grammar and punctuation errors.We will update this manual regularly.

Modification History

| Date | Content |
| --- | --- |
| 2008.10.22 | Create |
| 2009.02.12 | Add H264_DVR_SearchDevice for search device at LAN |
| 2009.02.18 | Add API with talk: H264_DVR_StartVoiceCom_MR，H264_DVR_VoiceComSendData，H264_DVR_StopVoiceCom，H264_DVR_SetTalkMode |
| 2009.09.26 | Add API below: H264_DVR_StartDVRRecord，H264_DVR_StopDVRRecord，H264_DVR_SetSystemDateTime， H264_DVR_GetDVRWorkState， H264_DVR_ClickKey |

Catalogue

1.	General Introduction	5

1.1 Introduction	5

1.2 Applicability	5

Desig of principia	6

1.3 Programming Description	6

1.4 API Calling Reference	7

2 Data Structure Description	9

2.1 Client Data Structure Description	9

2.1.1 Structure of device information	12

2.1.2 Date Information	12

2.1.3 Record File Information	12

2.1.4 Structure of Configuration	16

2.1.5 Network keyboard define	28

3 API Definition	31

3.1 SDK Initialization	31

3.2 Get alarm status	32

3.3 Device Registration	34

3.4 Real-time Monitor	35

3.5 Playback And Download	37

3.6 Playback Control	40

3.7 PTZ Control	40

3.8 System Configuration	41

3.9 Log Management	42

3.10 Remote Control	43

3.11 Upgrade	43

3.11 Audio Talk	45

3.12 Record Mode	47

3.13 Set System Time	47

3.14 Get Device status	48

3.15 Net Keyboard	48

# General Introduction

## Introduction

This SDK is for VANGUARD DVR,network video server and etc. The text describes functions and interfaces as well as relations among them.It also provide detailed demonstration.

This package includes two parts: operation and equipment management：

Operation

state listening, real-time monitor, real-time preview, character overlay, audio control, record playback and download, data storage, PTZ control, alarm deployment, voice dialogue, log management, user management, action on alarm, transparent com port and etc.

Device management

remote upgrade, remote restart/shutdown, equipment parameters setup such as general setting, alarm, record, serial port, image, log, user management, device timing, motion detection, network and etc.

The development package includes the following files：

| Network library | NetSDK | Head file |
| --- | --- | --- |
|  | NetSDK.lib | Lib file |
|  | NetSDK.dll | Interface library |
| Assitante library | H264Play.dll | Decode assistant library |
|  | DllDeinterlace.dll | Decode assistant library |
|  | hi_h264dec_w.dll | Decode assistant library |

## Applicability

- Support real-time monitor playback,、Alarm、Remote config、log management and etc。

- Support TCP network tramsmisson, Max 10 TCP connections

- You can use this SDK to connect to device to develop client-end program, or you can use SDK callback interface to develop server program such as stream media transfer, playback, alarm and etc.

- The client supports multi- image resolutions including QCIF、CIF、2CIF、HalfD1、D1，VGA（640×480）and etc.

- When SDK is playback/download, one logged  ID can not operate playback and downalod for the same channel at the same time.

- SDK performance has relationship woth device running status and client-end PC capability. Basically, it can support 1024 users to register at the same time, 1024-ch network preview and playback at the same time. Support 1024-ch alarm upload at the same time .And 100-ch video display

# Desig of principia

## Programming Description

- Initialization and clean up

- 1、First calling H264_DVR_Init() to initialize SDK，when Application exit, calling H264_DVR_Cleanup()to release all occupied resource.

- 2、 Most API shall call after H264_DVR_Init(), before H264_DVR_Cleanup() ，But H264_DVR_GetLastError can be called anywhere.

- Login in and login out

- Befor access device, you shall call H264_DVR_Login() login first, also you can call H264_DVR_LoginEx() to assign your client type logined. If login succeeded, will return a global handle, the handle is a session channel, you can use it to operate device. Call H264_DVR_Logout() to close this session.

- Callback Fuction

- Callback function have a parameter: dwUser, it is elf-define parameter, you can define your owner data.

## API Calling Reference

The below diagram describes a brief API calling reference for basic client application, and users can add in other function modules according to actual application requirements.

- Initialization

SDK Initialization H264_DVR_Init ()

- Set callback for alarm message

Set callback H264_DVR_SetDVRMessCallBack ()

- Login in

Login in H264_DVR_Login () H264_DVR_LoginEx () Subscribe for alarm message H264_DVR_SetupAlarmChan ()

- Operation of device ,get and set parameter configuration

Parameter configuration H264_DVR_GetDevConfig () H264_DVR_SetDevConfig () Log searching H264_DVR_FindDVRLog () PTZ control H264_DVR_PTZControl () H264_DVR_PTZControlEx ()

- Real-time preview

Open and close monitor channel H264_DVR_RealPlay () H264_DVR_StopRealPlay () Callback for saving monitor data H264_DVR_SetRealDataCallBack ()

- Playback/Download

Record searching H264_DVR_FindFile () Playback control H264_DVR_PlayBackByName() H264_DVR_PlayBackControl() H264_DVR_StopPlayBack() Download H264_DVR_GetFileByName () H264_DVR_GetDownloadPos() H264_DVR_StopGetFile ()

- Remote control

Upgrade H264_DVR_Upgrade() H264_DVR_GetUpgradeState() H264_DVR_CloseUpgradeHandle() Reboot /Clear log H264_DVR_ControlDVR（）

- Login out

Cancel subscribe for alarm message H264_DVR_CloseAlarmChan () Disconnect H264_DVR_Logout ()

- Release SDK resource

SDK exit H264_DVR_Cleanup ()

# Data Structure Description

## Client Data Structure Description

//PTZ control type

typedef enum PTZ_ControlType

{

TILT_UP = 0,			//UP

TILT_DOWN,			//DOWN

PAN_LEFT,				//LEFT

PAN_RIGTH,			//RIGTH

PAN_LEFTTOP,			//LEFT TOP

PAN_LEFTDOWN,			//LEFT DOWN

PAN_RIGTHTOP,			//RIGTH TOP

PAN_RIGTHDOWN,		//RIGTH DOWN

ZOOM_IN,				//ZOOM IN

ZOOM_OUT,				//ZOOM OUT

FOCUS_FAR,			//FOCUS FAR

FOCUS_NEAR,			//FOCUS NEAR

IRIS_OPEN,			//IRIS OPEN

IRIS_CLOSE,			//IRIS CLOSE

EXTPTZ_OPERATION_ALARM,			//ALARM

EXTPTZ_LAMP_ON,					//LIGTH OPEN

EXTPTZ_LAMP_OFF,					//LIGTH CLOSE

EXTPTZ_POINT_SET_CONTROL,		//SET PRESET POINT

EXTPTZ_POINT_DEL_CONTROL,		//CLEAR PRESET POINT

EXTPTZ_POINT_MOVE_CONTROL,		//GOTO PRESET POINT

EXTPTZ_STARTPANCRUISE,			//START PAN CRUISE

EXTPTZ_STOPPANCRUISE,			//STOP PAN CRUISE	EXTPTZ_SETLEFTBORDER,			//SET LEFT BORDER

EXTPTZ_SETRIGHTBORDER,			//SET RIGHT BORDER

EXTPTZ_STARTLINESCAN,			//START AUTO SCAN

EXTPTZ_CLOSELINESCAN,			//STOP AUTO SCAN

EXTPTZ_ADDTOLOOP,				//ADD PRESET POINT TO CRUISE LINE

EXTPTZ_DELFROMLOOP,				//DEL PRESET POINT FROM CRUISE LINE

EXTPTZ_POINT_LOOP_CONTROL,		//START CRUISE

EXTPTZ_POINT_STOP_LOOP_CONTROL,//STOP CRUISE

EXTPTZ_CLOSELOOP,				//CLEAR CRUISE LINE

EXTPTZ_FASTGOTO,					//FAST GOTO

EXTPTZ_AUXIOPEN,				//AUX OPEN

EXTPTZ_OPERATION_MENU,		//OPERATION MENU

EXTPTZ_REVERSECOMM,			//REVER CAMERAL

EXTPTZ_OPERATION_RESET,		///< PTZ RESET

EXTPTZ_TOTAL,

};

Error code，return by GetLastError

typedef enum SDK_RET_CODE

{

H264_DVR_NOERROR					= 0,		//no error

H264_DVR_SUCCESS					= 1,		//success

H264_DVR_SDK_NOTVALID			= -10000,	//invalid request

H264_DVR_NO_INIT					= -10001,	//SDK not inited

H264_DVR_ILLEGAL_PARAM			= -10002,	// illegal user parameter

H264_DVR_INVALID_HANDLE			= -10003,	//handle is null

H264_DVR_SDK_UNINIT_ERROR		= -10004,	//SDK clear error

H264_DVR_SDK_TIMEOUT				= -10005,	//timeout

H264_DVR_SDK_MEMORY_ERROR		= -10006,	//memory error	H264_DVR_SDK_NET_ERROR			= -10007,	//network error

H264_DVR_SDK_OPEN_FILE_ERROR	= -10008,	//open file fail

H264_DVR_SDK_UNKNOWNERROR		= -10009,	//unknown error

H264_DVR_DEV_VER_NOMATCH			= -11000,	//version mismatch

H264_DVR_ERROR_GET_DATA			= -11001,	//get data fail（including configure, user information and etc）

H264_DVR_OPEN_CHANNEL_ERROR		= -11200,	//open channel fail

H264_DVR_CLOSE_CHANNEL_ERROR	= -11201,	//close channel fail

H264_DVR_SUB_CONNECT_ERROR		= -11202,	//open media connet fail	H264_DVR_SUB_CONNECT_SEND_ERROR	= -11203,	//media connet send data fail

/// error code of user management

H264_DVR_NOPOWER					= -11300,	//no power

H264_DVR_PASSWORD_NOT_VALID		= -11301,	// password not valid

H264_DVR_LOGIN_USER_NOEXIST		= -11302,	// user not exist

H264_DVR_USER_LOCKED				= -11303,	// user is locked

H264_DVR_USER_IN_BLACKLIST		= -11304,	// user is in backlist

H264_DVR_USER_HAS_USED			= -11305,	// user have logined	H264_DVR_USER_NOT_LOGIN			= -11305,	// no login	H264_DVR_CONNECT_DEVICE_ERROR   = -11306,	// maybe device no exist

/// error code of configure management

H264_DVR_OPT_RESTART				= -11400,	// need to restart application	H264_DVR_OPT_REBOOT				= -11401,	// need to reboot system	H264_DVR_OPT_FILE_ERROR			= -11402,	// write file fail

H264_DVR_OPT_CAPS_ERROR			= -11403,	// not support

H264_DVR_OPT_VALIDATE_ERROR		= -11404,	// validate fail	H264_DVR_OPT_CONFIG_NOT_EXIST	= -11405,	// config not exist

H264_DVR_CTRL_PAUSE_ERROR		= -11500,	// pause fail

};

- Alarm Event Type

enum SDK_EventCodeTypes

{

SDK_EVENT_CODE_INIT = 0,

SDK_EVENT_CODE_LOCAL_ALARM = 1,	// local alarm	SDK_EVENT_CODE_NET_ALARM,			// network alarm	SDK_EVENT_CODE_MANUAL_ALARM,		// manual alarm

SDK_EVENT_CODE_VIDEO_MOTION,		// motion detect

SDK_EVENT_CODE_VIDEO_LOSS,			// loss detect	SDK_EVENT_CODE_VIDEO_BLIND,		// blind detect

SDK_EVENT_CODE_VIDEO_TITLE,

SDK_EVENT_CODE_VIDEO_SPLIT,

SDK_EVENT_CODE_VIDEO_TOUR,

SDK_EVENT_CODE_STORAGE_NOT_EXIST,

SDK_EVENT_CODE_STORAGE_FAILURE,

SDK_EVENT_CODE_LOW_SPACE,

SDK_EVENT_CODE_NET_ABORT,

SDK_EVENT_CODE_COMM,

SDK_EVENT_CODE_STORAGE_READ_ERROR,

SDK_EVENT_CODE_STORAGE_WRITE_ERROR,

SDK_EVENT_CODE_NET_IPCONFLICT,

SDK_EVENT_CODE_ALARM_EMERGENCY,

SDK_EVENT_CODE_DEC_CONNECT,

SDK_EVENT_CODE_NR,

};

//alarm information

typedef struct SDK_ALARM_INFO

{

int nChannel;

int iEvent;  ///< refer to SDK_EventCodeTypes

int iStatus; ///< 0: start 1: stop

SDK_SYSTEM_TIME SysTime;

}SDK_AlarmInfo;

### Structure of device information

- Structure of device define as below

typedef struct _H264_DVR_DEVICEINFO

{

char sSoftWareVersion[64];	///< software version

char sHardWareVersion[64];	///< hardware version

char sEncryptVersion[64];	///< encrypt version

SDK_SYSTEM_TIME tmBuildTime;///< build time

char sSerialNumber[64];			///< device serial number

int byChanNum;				///< channel number of video in

int iVideoOutChannel;		///< channel number of video out

int byAlarmInPortNum;		///< channel number of alarm in

int byAlarmOutPortNum;		///< channel number of alarm out

int iTalkInChannel;			///< channel number of talk in

int iTalkOutChannel;		///< channel number of talk out

int iExtraChannel;	         ///< channel number of extra

int iAudioInChannel;		     ///< channel number of audio in

int iCombineSwitch;			///< channel number of combine

}H264_DVR_DEVICEINFO,*LPH264_DVR_DEVICEINFO;

### Date Information

typedef struct SDK_SYSTEM_TIME{

int  year;///< year

int  month;///< month，January = 1, February = 2, and so on.

int  day;///< day

int  wday;///< week, Sunday = 0, Monday = 1, and so on

int  hour;///< hour

int  minute;///< minute

int  second;///< second

int  isdst;///< DST(daylight saving time) flag, Yes = 1, No = 0

}SDK_SYSTEM_TIME;

### Record File Information

//search condition structure

typedef struct

{

int nChannelN0;			//channel NO, start with 0

int nFileType;			//record type

H264_DVR_TIME startTime;	//start time

H264_DVR_TIME endTime;	//end time

char szCard[32];			//card number

}H264_DVR_FINDINFO;

//the return of record information structure

typedef struct

{

int ch;					///< channel NO, start with 0

int size;					///< record size(BYTE)

char sFileName[108];		///< record file name

SDK_SYSTEM_TIME stBeginTime;	///< start time of record	SDK_SYSTEM_TIME stEndTime;		///< end time of record

}H264_DVR_FILE_DATA;

- Protocol Of Serial Information

struct SDK_COMMATTRI

{

int	iDataBits;	// data bit: [5,8]

int	iStopBits;	// stop bit: [0,2]

int	iParity;	// parity: 0: None 1: odd 2：even 3: mark 4: space

int	iBaudRate;	// baudrate: 115200,57600,38400,9600,4800,2400 and so on

};

// serial configure

struct SDK_CONFIG_COMM_X

{

char iProtocolName[32];	// Protocol:“Console”

int iPortNo;		// Port No.

SDK_COMMATTRI aCommAttri;		// attribute of serial

};

- 云台协议

struct SDK_STR_CONFIG_PTZ

{

char sProtocolName[NET_MAX_PTZ_PROTOCOL_LENGTH];	// Protocol

int	ideviceNo;				// PTZ device NO.

int	iNumberInMatrixs;		// NO. in matrixs

int iPortNo;				// serial port NO.:	[1, 4]

SDK_COMMATTRI dstComm;			// attribute of serial

};

//all channel of PTZ protocol

struct SDK_STR_PTZCONFIG_ALL

{

SDK_STR_CONFIG_PTZ ptzAll[NET_MAX_CHANNUM];

};

- User Manager Data Structure

Right List

typedef struct _OPR_RIGHT

{

string		name;

}OPR_RIGHT;

typedef struct _USER_INFO

{

int			rigthNum;

string		rights[NET_MAX_RIGTH_NUM];

string		strGroupname;

string		strmemo;

string		strname;

string		strpassWord;

bool		reserved;		//is reserved user

bool		shareable;		//is shareable

}USER_INFO;

typedef struct _USER_GROUP_INFO

{

int			rigthNum;

string		memo;

string		name;

string		rights[NET_MAX_RIGTH_NUM];	//right list

}USER_GROUP_INFO;

//all of user and group information structure

typedef struct _USER_MANAGE_INFO

{

int					rigthNum;

OPR_RIGHT			rightList[NET_MAX_RIGTH_NUM];

int					groupNum;

USER_GROUP_INFO		groupList[NET_MAX_GROUP_NUM];

int					userNum;

USER_INFO			userList[NET_MAX_USER_NUM];

}USER_MANAGE_INFO;

//modify user

typedef struct _CONF_MODIFYUSER

{

std::string sUserName;

USER_INFO User;

}CONF_MODIFYUSER;

//modify group

typedef struct _CONF_MODIFYGROUP

{

std::string sGroupName;

USER_GROUP_INFO Group;

}CONF_MODIFYGROUP;

/// modify password

struct _CONF_MODIFY_PSW

{

std::string sUserName;

std::string sPassword;

std::string sNewPassword;

};

- Log Information

#define NET_MAX_RETURNED_LOGLIST	1024		//the max item of Log

/// log search condition

struct SDK_LogSearchCondition

{

int nType;	///< log type: 0: all 1: system 2: configure 3: storage 4: alarm 5：record 6: account 7: file

int iLogPosition;			///< return of last log item position in the whole logs

SDK_SYSTEM_TIME stBeginTime;	///< begin time

SDK_SYSTEM_TIME stEndTime;		///< end time

};

//return of Log search

struct SDK_LogList

{

int iNumLog;

struct LogItem

{

char sType[24];	///< log type

char sUser[32];	///< Operator of log

char sData[68];	///< log data

SDK_SYSTEM_TIME stLogTime;	///< the time of log happened

} Logs[NET_MAX_RETURNED_LOGLIST];

};

- Storage information structure

struct SDK_STORAGEDISK

{

int		iPhysicalNo;

int		iPartNumber;

SDK_DriverInformation diPartitions[SDK_MAX_DRIVER_PER_DISK];

};

struct SDK_StorageDeviceInformationAll

{

int iDiskNumber;

SDK_STORAGEDISK vStorageDeviceInfoAll[SDK_MAX_DISK_PER_MACHINE];

};

- Real-Time Monitor

typedef struct{

int nChannel;	//Channel NO.

int nStream;	//0: main stream，1: extra stream

int nMode;	//0：TCP, 1：UDP

}H264_DVR_CLIENTINFO,*LPH264_DVR_CLIENTINFO;

### Structure of Configuration

Commands of H264_DVR_GetDevConfig、H264_DVR_SetDevConfig

typedef enum _SDK_CONFIG_TYPE

{

E_SDK_CONFIG_NOTHING = 0,

//User Managerment

E_SDK_CONFIG_USER,           //User information，including power list，user list and group list

USER_MANAGE_INFO

E_SDK_CONFIG_ADD_USER,		//add user USER_INFO

E_SDK_CONFIG_MODIFY_USER,	//modify user  CONF_MODIFYUSER

E_SDK_CONFIG_DELETE_USER,  //del user  USER_INFO

E_SDK_CONFIG_ADD_GROUP,		//add group   USER_GROUP_INFO

E_SDK_CONFIG_MODIFY_GROUP,	//modify group    CONF_MODIFYGROUP

E_SDK_COFIG_DELETE_GROUP,  //del group    USER_GROUP_INFO

E_SDK_CONFIG_MODIFY_PSW,	//modify password  _CONF_MODIFY_PSW

//device ability

E_SDK_CONFIG_ABILITY_SYSFUNC = 9,//support network services   SDK_SystemFunctio

E_SDK_CONFIG_ABILTY_ENCODE,		//encode ability  CONFIG_EncodeAbility

E_SDK_CONFIG_ABILITY_PTZPRO,		//protocols of ptz support   SDK_PTZPROTOCOLFUNC

E_SDK_COMFIG_ABILITY_COMMPRO,		// protocols of 232 support        SDK_COMMFUNC

E_SDK_CONFIG_ABILITY_MOTION_FUNC,	//Motion detect   SDK_MotionDetectFunction

E_SDK_CONFIG_ABILITY_BLIND_FUNC,	//Blind detect     SDK_BlindDetectFunction

E_SDK_CONFIG_ABILITY_DDNS_SERVER,	// type  of DDNS services support SDK_DDNSServiceFunction

E_SDK_CONFIG_ABILITY_TALK,			// encode type of audio talk support

//Device configuration

E_SDK_CONFIG_SYSINFO = 17,		//system information  H264_DVR_DEVICEINFO

E_SDK_CONFIG_SYSNORMAL,			//general      SDK_CONFIG_NORMAL

E_SDK_CONFIG_SYSENCODE,			//encode       SDK_EncodeConfigAll

E_SDK_CONFIG_SYSNET,			//network       SDK_CONFIG_NET_COMMON

E_SDK_CONFIG_PTZ,				//ptz		SDK_STR_PTZCONFIG_ALL

E_SDK_CONFIG_COMM,				//232		SDK_CommConfigAll

E_SDK_CONFIG_RECORD,			//record	SDK_RECORDCONFIG_ALL

E_SDK_CONFIG_MOTION,			//motion detect SDK_MOTIONCONFIG

E_SDK_CONFIG_SHELTER,		//blind detect	SDK_BLINDDETECTCONFIG_ALL

E_SDK_CONFIG_VIDEO_LOSS,  	//loss detect		SDK_VIDEOLOSSCONFIG_ALL

E_SDK_CONFIG_ALARM_IN,		//alarm in			SDK_ALARM_INPUTCONFIG_ALL

E_SDK_CONFIG_ALARM_OUT,		//alarm out

E_SDK_CONFIG_DISK_MANAGER	//disk management

E_SDK_CONFIG_OUT_MODE,		//out mode

E_SDK_CONFIG_AUTO,			//auto maintain	SDK_AutoMaintainConfig

E_SDK_CONFIG_DEFAULT,     	//set default

E_SDK_CONFIG_DISK_INFO,		//disk info		SDK_StorageDeviceInformationAll

E_SDK_CONFIG_LOG_INFO,		//get log		SDK_LogList

E_SDK_CONFIG_NET_IPFILTER,	//network services：black/white list	SDK_NetIPFilterConfig

E_SDK_CONFIG_NET_DHCP,		//network services：DHCP

E_SDK_CONFIG_NET_DDNS,		//network services：DDNS		SDK_NetDDNSConfigALL

E_SDK_CONFIG_NET_EMAIL,		//network services：EMAIL		SDK_NetEmailConfig

E_SDK_CONFIG_NET_MULTICAST, //network services:Multicast		SDK_NetMultiCastConfig

E_SDK_CONFIG_NET_NTP,		//network services： NTP		SDK_NetNTPConfig

E_SDK_CONFIG_NET_PPPOE,		//network services:  PPPOE		SDK_NetPPPoEConfig

E_SDK_CONFIG_NET_DNS,		//network services:  DNS		SDK_NetDNSConfig

E_SDK_CONFIG_NET_FTPSERVER, //network services: FTP		SDK_FtpServerConfig

E_SDK_CONFIG_SYS_TIME,	//system time

E_SDK_CONFIG_CLEAR_LOG,		//clear log

E_SDK_REBOOT_DEV,			//reboot device

E_SDK_CONFIG_ABILITY_LANG,	          //languages support

E_SDK_CONFIG_VIDEO_FORMAT,            //Video format

E_SDK_CONFIG_COMBINEENCODE,	          //combine-encode

E_SDK_CONFIG_EXPORT,	               //config export

E_SDK_CONFIG_IMPORT,	               //config import

E_SDK_LOG_EXPORT,		               //log export

E_SDK_CONFIG_COMBINEENCODEMODE,       //mode of combine-encode

E_SDK_WORK_STATE,	                    //work status

}SDK_CONFIG_TYPE;

/// type of DDNS support

struct SDK_DDNSServiceFunction

{

int  nTypeNum;

char vDDNSType[NET_MAX_DDNS_TYPE][64];

};

/// blind detect support

struct SDK_BlindDetectFunction

{

int iBlindCoverNum;	///< the number of cover area support

};

/// motion detect

struct SDK_MotionDetectFunction

{

int iGridRow;  ///< the number of row

int iGridColumn; ///< the number of colum

};

/// protocols of 232 support

struct SDK_COMMFUNC

{

int nProNum;      ///< the numbers of protocol

char vCommProtocol[SDK_COM_TYPES][32]; ///< the name of protocol

};

/// protocols of PTZ

struct SDK_PTZPROTOCOLFUNC

{

int nProNum;

char vPTZProtocol[100][NET_MAX_PTZ_PROTOCOL_LENGTH];

};

/// encode information

struct SDK_EncodeInfo

{

bool bEnable;			///< enable

int iStreamType;		///< stream type see refer to capture_channel_t

bool bHaveAudio;		///< is support audio

unsigned int uiCompression;		///< mask of capture_comp_t

unsigned int uiResolution;		///< mask of capture_size_t

};

/// encode power

struct CONFIG_EncodeAbility

{

int iMaxEncodePower;		///< max encode power

SDK_EncodeInfo vEncodeInfo[SDK_CHL_FUNCTION_NUM];	///< encode information

SDK_EncodeInfo vCombEncInfo[SDK_CHL_FUNCTION_NUM];	///< combine-encode information

};

///system function

struct SDK_SystemFunction

{

bool vEncodeFunction[SDK_ENCODE_FUNCTION_TYPE_NR];	///< Encode Functions

bool vAlarmFunction[SDK_ALARM_FUNCTION_TYPE_NR];	///< Alarm Fucntions

bool vNetServerFunction[SDK_NET_SERVER_TYPES_NR];	///< Net Server Functions

bool vPreviewFunction[SDK_PREVIEW_TYPES_NR];		///< Preview Functions

};

///< Auto-Maintain setting

struct SDK_AutoMaintainConfig

{

int iAutoRebootDay;			///< interval of Auto-Reboot days

int iAutoRebootHour;			///< time to reboot [0, 23]

int iAutoDeleteFilesDays;		///< interval of Auto-Del record file [0, 30]

};

//Disk info

struct SDK_STORAGEDISK

{

int		iPhysicalNo;       // Physical No.

int		iPartNumber;		// Partition numbers

SDK_DriverInformation diPartitions[SDK_MAX_DRIVER_PER_DISK];

};

struct SDK_StorageDeviceInformationAll

{

int iDiskNumber;

SDK_STORAGEDISK vStorageDeviceInfoAll[SDK_MAX_DISK_PER_MACHINE];

};

// Type of PTZ link

enum PtzLinkTypes

{

PTZ_LINK_NONE,		// NONE

PTZ_LINK_PRESET,		// GOTO PRESET

PTZ_LINK_TOUR,		// TOUR

PTZ_LINK_PATTERN		// PATTERN

};

// PTZ Link Config

struct SDK_PtzLinkConfig

{

int iType;		// see refer to PtzLinkTypes

int iValue;		// value of link type

};

// handler of event

struct SDK_EventHandler

{

unsigned int	dwRecord;				// bitmask of record. Bit per channel

unsigned int	iRecordLatch;			// record latch：10～300 sec.

unsigned int	dwTour;					// bitmask of tour. Bit per channel

unsigned int	dwSnapShot;				// bitmask of snapshot. Bit per channel

unsigned int	dwAlarmOut;				// bitmask of alarm out. Bit per channel

unsigned int	dwMatrix;				// bitmask of matrix. Bit per channel

int		iEventLatch;			// interval of event(unit:sec.)

int		iAOLatch;				// Alarm out latch：10～300 sec

SDK_PtzLinkConfig PtzLink[NET_MAX_CHANNUM];		// PTZ link activation

SDK_CONFIG_WORKSHEET schedule;		// weeksheet of record

bool	bRecordEn;				// enable flag of record

bool	bTourEn;				// enable flag of tour

bool	bSnapEn;				// enable flag of snapshot

bool	bAlarmOutEn;			// enable flag of alarm out

bool	bPtzEn;				// enable flag of PTZ link

bool	bTip;				// enable flag of screen tip

bool	bMail;				// enable flag of sending email

bool	bMessage;				// enable flag of sending message to alarm center

bool	bBeep;					// enable flag of buzzer beep

bool	bVoice;					// enable flag of voice tip

bool	bFTP;					// enable flag of FTP unload

bool	bMatrixEn;				// no used

bool	bLog;					// enable flag of log

bool	bMessagetoNet;			// no used

};

///< Blind detect

struct SDK_BLINDDETECTCONFIG

{

bool	bEnable;		          ///< enable

int	iLevel;			     ///< sensitivity：1～6

SDK_EventHandler hEvent;	///< handler of blind detect event

};

/// All channel of blind detect configuration

struct SDK_BLINDDETECTCONFIG_ALL

{

SDK_BLINDDETECTCONFIG vBlindDetectAll[NET_MAX_CHANNUM];

};

///< Alarm in

struct SDK_ALARM_INPUTCONFIG

{

bool	bEnable;		          ///< enable

int		iSensorType;	     ///< Sensor Type: Normal Open or Normal Close

SDK_EventHandler hEvent;	///< handler of alarm in

};

///< All channel of alarm in configuration

struct SDK_ALARM_INPUTCONFIG_ALL

{

SDK_ALARM_INPUTCONFIG vAlarmConfigAll[NET_MAX_CHANNUM];

};

///< Motion detect

struct SDK_MOTIONCONFIG

{

bool bEnable;							     //< enable

int iLevel;								//< sensitivity：[1,6]

unsigned int mRegion[NET_MD_REGION_ROW];		//< regions of motion detect，one bit per column, Max region: 18*22

SDK_EventHandler hEvent;					//< handler of motion detect

};

/// All channel of video motion configuration

struct SDK_MOTIONCONFIG_ALL

{

SDK_MOTIONCONFIG vMotionDetectAll[NET_MAX_CHANNUM];

};

///< video loss detect

struct SDK_VIDEOLOSSCONFIG

{

bool bEnable;			///< enable

SDK_EventHandler hEvent;	///< event handler

};

/// All channel of video loss configuration

struct SDK_VIDEOLOSSCONFIG_ALL

{

SDK_VIDEOLOSSCONFIG vGenericEventConfig[NET_MAX_CHANNUM];

};

/// record mode type

enum SDK_RecordModeTypes

{

SDK_RECORD_MODE_CLOSED,		///< Closed

SDK_RECORD_MODE_MANUAL,		///< Manual: record all the time

SDK_RECORD_MODE_CONFIG,		///< Configuration: according to SDK_RECORDCONFIG

SDK_RECORD_MODE_NR,

};

///< record setting

struct SDK_RECORDCONFIG

{

int iPreRecord;			 ///< pre-record time（Unit:sec.）

bool bRedundancy;		     ///< redundancy record

bool bSnapShot;			///< no used

int iPacketLength;		    ///< record length（unit:minute）[1, 255]

int iRecordMode;		    ///< record mode，refer to SDK_RecordModeTypes

SDK_CONFIG_WORKSHEET wcWorkSheet;			///< worksheet

unsigned int typeMask[NET_N_WEEKS][NET_N_TSECT];		///< mask of record type, corresponding to worksheet

};

// All channel of record configuration

struct SDK_RECORDCONFIG_ALL

{

SDK_RECORDCONFIG vRecordConfigAll[NET_MAX_CHANNUM];

};

// General Configuration

typedef struct _SDK_CONFIG_NORMAL

{

NEW_NET_TIME sysTime;		 ///< system time

int iLocalNo;			     ///< device No.:[0, 998]

int iOverWrite;			///< when disk full, 0: OverWrite, 1: StopRecord

int iSnapInterval;			///< no used

char sMachineName[64];	    ///< device name

int iVideoStartOutPut;	    ///< no used

int iAutoLogout;		    ///< auto logout [0, 120], 0 means never

int iVideoFormat;		    ///< video format: 0:PAL, 1:NTSC, 2:SECAM

int iLanguage;			///< language: 0:English, 1: SimpChinese, 2:TradChinese, 3: Italian, 4:Spanish, 5:Japanese, 6:Russian, 7:French, 8:German

int iDateFormat;		///< date format: 0:YYMMDD, 1:MMDDYY, 2:DDMMYY

int iDateSeparator;		///< Date separator: 0: ., 1: -, 2: /

int iTimeFormat;		///< Time format: 0: 12, 1: 24

int iDSTRule;			///< DST rule: 0: OFF, 1: ON

int iWorkDay;			///< work day

DSTPoint dDSTStart;

DSTPoint dDSTEnd;

}SDK_CONFIG_NORMAL;

// encode configuration

struct SDK_CONFIG_ENCODE

{

SDK_MEDIA_FORMAT dstMainFmt[SDK_ENCODE_TYPE_NUM];		//  main stream

SDK_MEDIA_FORMAT dstExtraFmt[SDK_EXTRATYPES];	         //  Extra stream

SDK_MEDIA_FORMAT dstSnapFmt[SDK_ENCODE_TYPE_NUM];		//  Snapshot

};

// all of channel encode configuration

struct  SDK_EncodeConfigAll

{

SDK_CONFIG_ENCODE vEncodeConfigAll[NET_MAX_CHANNUM];

};

// combine-encode

struct  SDK_CombineEncodeConfigAll

{

SDK_CONFIG_ENCODE vEncodeConfigAll[NET_MAX_COMBINE_NUM];

};

// newwork configuration

struct SDK_CONFIG_NET_COMMON

{

char HostName[NET_NAME_PASSWORD_LEN]; ///< hostname

CONFIG_IPAddress HostIP;   			///< IP

CONFIG_IPAddress Submask; 			///< Netmask

CONFIG_IPAddress Gateway;			///< NetGateway

int HttpPort;						///< HTTP port

int TCPPort;						///< TCP port

int SSLPort;						///< no used

int UDPPort;						///< no used

int MaxConn;						///< max connect

int MonMode;						///< translation protocol: 0:TCP, 1: UDP, 2: MCAST, only support TCP now

int MaxBps;					///< no used

int TransferPlan;				///< Translation policy: 0: AUTO 1: Quality first 2: fluency first

bool bUseHSDownLoad;			///< flag of high speed download

};

// PTZ configuration

struct SDK_STR_CONFIG_PTZ

{

char sProtocolName[NET_MAX_PTZ_PROTOCOL_LENGTH];	//< Protocol name

int	ideviceNo;			//< device No.

int	iNumberInMatrixs;		//< No. in Matrixs

int iPortNo;				//< Port No.[1, 4]

SDK_COMMATTRI dstComm;		//< comm attribute

};

// all channel of PTZ configuration

struct SDK_STR_PTZCONFIG_ALL

{

SDK_STR_CONFIG_PTZ ptzAll[NET_MAX_CHANNUM];

};

// 232 configuration

struct SDK_CONFIG_COMM_X

{

char iProtocolName[32];	// Protocol name:“Console”

int iPortNo;		// Port No.

SDK_COMMATTRI aCommAttri;		// comm attribute

};

// all channel of 232 configuration

struct SDK_CommConfigAll

{

SDK_CONFIG_COMM_X vCommConfig[SDK_COM_TYPES];

};

///< IP Fliter

struct SDK_NetIPFilterConfig

{

bool Enable;		///< enable

CONFIG_IPAddress BannedList[NET_MAX_FILTERIP_NUM];		///< black list

CONFIG_IPAddress TrustList[NET_MAX_FILTERIP_NUM];		///< white list

};

///< multicast

struct SDK_NetMultiCastConfig

{

bool Enable;		///< enable

SDK_RemoteServerConfig Server;		///< multicast server

};

///< pppoe

struct SDK_NetPPPoEConfig

{

bool Enable;	///< enable

SDK_RemoteServerConfig Server;		///< PPPOE server

CONFIG_IPAddress addr;		///< ip get from PPPOE dial

};

///< DDNS

struct SDK_NetDDNSConfig

{

bool Enable;	///< enable

char DDNSKey[NET_NAME_PASSWORD_LEN];	///< the type of DDNS name

char HostName[NET_NAME_PASSWORD_LEN];	///< hostname

SDK_RemoteServerConfig Server;		///< DDNS server

};

///< DDNS

struct SDK_NetDDNSConfigALL

{

SDK_NetDDNSConfig ddnsConfig[5];

};

///< FTP

struct SDK_FtpServerConfig {

///< enable

bool Enable;

///< FTP server

SDK_RemoteServerConfig Server;

///< spare server IP

CONFIG_IPAddress SpareIP;

///< path name in FTP server

char RemotePathName[NET_MAX_PATH_LENGTH];

///< max file lenght

int  FileMaxLen;

///< upload periods

SDK_TIMESECTION  UpLoadPeriod[NET_N_MIN_TSECT];

};

///< NTP

struct SDK_NetNTPConfig

{

///< enable

bool Enable;

///< NTP server

SDK_RemoteServerConfig Server;

///< update period

int UpdatePeriod;

///< time zone

int TimeZone;

};

#define  NET_MAX_EMAIL_TITLE_LEN 64

#define  NET_MAX_EMAIL_RECIEVERS  5

#define  NET_EMAIL_ADDR_LEN  32

///< EMAIL

struct SDK_NetEmailConfig

{

///< enalbe

bool Enable;

///< smtp server

SDK_RemoteServerConfig Server;

///< is need SSL ?

bool bUseSSL;

///< sender address

char SendAddr[NET_EMAIL_ADDR_LEN];

///< receiver

char Recievers[NET_MAX_EMAIL_RECIEVERS][NET_EMAIL_ADDR_LEN];

///< email title

char Title[NET_MAX_EMAIL_TITLE_LEN];

///< time section

SDK_TIMESECTION Schedule[NET_N_MIN_TSECT];

};

///< DNS

struct SDK_NetDNSConfig

{

CONFIG_IPAddress		PrimaryDNS;

CONFIG_IPAddress		SecondaryDNS;

};

/// audio format for audio talk

struct SDK_AudioInFormatConfig

{

int iBitRate;	   ///< bitrate，(unit:kbps)

int iSampleRate;   ///< sample rate(unit:Hz)

int iSampleBit;    ///< sample bit

int iEncodeType;   ///< type of encode，see refer to AudioEncodeTypes

};

/// alarm status

struct SDK_DVR_ALARMSTATE

{

int iVideoMotion; ///< motion detect status, bit mask for channel,bit0 means channel 1,and so on,1: alarming 0: normal

int iVideoBlind; ///< blind detect status, bit mask for channel,bit0 means channel 1,and so on,1: alarming 0: normal

int iVideoLoss;	///< loss detect status, bit mask for channel,bit0 means channel 1,and so on,1: alarming 0: normal

int iAlarmIn;	///< alarm in status, bit mask for channel,bit0 means channel 1,and so on,1: alarming 0: normal

int iAlarmOut;	///< alarm out status, bit mask for channel,bit0 means channel 1,and so on,1: alarming 0: normal

};

// channel status

struct SDK_DVR_CHANNELSTATE

{

bool bRecord; ///< is recording

int iBitrate;	///< bitrate

};

// device work status

struct SDK_DVR_WORKSTATE

{

SDK_DVR_CHANNELSTATE vChnState[NET_MAX_CHANNUM];

SDK_DVR_ALARMSTATE vAlarmState;

};

### Network keyboard define

/// KEY VALUE

enum SDK_NetKeyBoardValue

{

SDK_NET_KEY_0, SDK_NET_KEY_1, SDK_NET_KEY_2, SDK_NET_KEY_3, SDK_NET_KEY_4, SDK_NET_KEY_5, SDK_NET_KEY_6, SDK_NET_KEY_7, SDK_NET_KEY_8, SDK_NET_KEY_9,

SDK_NET_KEY_10, SDK_NET_KEY_11, SDK_NET_KEY_12, SDK_NET_KEY_13, SDK_NET_KEY_14, SDK_NET_KEY_15, SDK_NET_KEY_16, SDK_NET_KEY_10PLUS,

SDK_NET_KEY_UP = 20,     // UP

SDK_NET_KEY_DOWN,        // DOWN

SDK_NET_KEY_LEFT,        // LEFT

SDK_NET_KEY_RIGHT,       // RIGHT

SDK_NET_KEY_SHIFT,

SDK_NET_KEY_PGUP,        // PAGE UP

SDK_NET_KEY_PGDN,        // PAGE DOWN

SDK_NET_KEY_RET,         // ENTER

SDK_NET_KEY_ESC,         // ESC

SDK_NET_KEY_FUNC,        // FUNC

SDK_NET_KEY_PLAY,        // PLAY/PAUSE

SDK_NET_KEY_BACK,        // BACK

SDK_NET_KEY_STOP,        // STOP

SDK_NET_KEY_FAST,        // FAST

SDK_NET_KEY_SLOW,        // SLOW

SDK_NET_KEY_NEXT,        // NEXT FILE

SDK_NET_KEY_PREV,        // PREV FILE

SDK_NET_KEY_REC = 40,    // ENTER RECORD SETTING PAGE

SDK_NET_KEY_SEARCH,      // ENTER RECORD SEARCH PAGE

SDK_NET_KEY_INFO,        // ENTER SYSTEM INFO PAGE

SDK_NET_KEY_ALARM,       // ENTER ALARM OUT PAGE

SDK_NET_KEY_ADDR,        // ENTER REMOTE ADDRESS SETTING PAGE

SDK_NET_KEY_BACKUP,      // ENTER BACKUP PAGE

SDK_NET_KEY_SPLIT,       // NEXT SPLIT MODE

SDK_NET_KEY_SPLIT1,      // SLPIT MODE 1

SDK_NET_KEY_SPLIT4,      // SLPIT MODE 4

SDK_NET_KEY_SPLIT8,      // SLPIT MODE 8

SDK_NET_KEY_SPLIT9,      // SLPIT MODE 9

SDK_NET_KEY_SPLIT16,     // SLPIT MODE 16

SDK_NET_KEY_SHUT,        // SHUTDOWN

SDK_NET_KEY_MENU,        // MENU

SDK_NET_KEY_PTZ = 60,    // ENTER PTZ CONTROL PAGE

SDK_NET_KEY_TELE,        // ZOOM -

SDK_NET_KEY_WIDE,        // ZOOM +

SDK_NET_KEY_IRIS_SMALL,  // APERTURE -

SDK_NET_KEY_IRIS_LARGE,  // APERTURE +

SDK_NET_KEY_FOCUS_NEAR,  // FOCUS -

SDK_NET_KEY_FOCUS_FAR,   // FOCUS +

SDK_NET_KEY_BRUSH,       // BRUSH

SDK_NET_KEY_LIGHT,       // LIGHT

SDK_NET_KEY_SPRESET,     // SET PRESET POINT

SDK_NET_KEY_GPRESET,     // GOTO PRESET POINT

SDK_NET_KEY_DPRESET,     // CLEAR PRESET POINT

SDK_NET_KEY_PATTERN,     // PATTERN

SDK_NET_KEY_AUTOSCAN,    // AUTO-SCAN ON/OFF

SDK_NET_KEY_AUTOTOUR,    // AUTO-TOUR ON/OFF

SDK_NET_KEY_AUTOPAN,     // AUTO-PAN ON/OFF

};

/// keyboard status

enum SDK_NetKeyBoardState

{

SDK_NET_KEYBOARD_KEYDOWN,	// key down

SDK_NET_KEYBOARD_KEYUP,		// key up

};

struct SDK_NetKeyBoardData

{

int iValue;                 // see refer to SDK_NetKeyBoardValue

int iState;                 // see refer to SDK_NetKeyBoardState

};

# API Definition

## SDK Initialization

- H264_DVR_API long H264_DVR_GetLastError();;

- API description: It is to return function failure code. when you failed to call the following interface, you can call this fucntion to get error code.

- Parameter: none

- Return: Please see refer to error code

- Reference API：

typedef void (__stdcall *fDisConnect)(long lLoginID, char *pchDVRIP, long nDVRPort, unsigned long dwUser);

- H264_DVR_API long H264_DVR_Init(fDisConnect cbDisConnect, unsigned long dwUser);

- API description：Initialize SDK, calling before all SDK function

- Parameter：

- cbDisConnect

- Disconnect callback function. It is to callback disconnect device excluding device logout successfully(call H264_DVR_Logout(), set it as 0 when forbid callbacking.

- [in]dwUser

- User data

- CallBack function Parameters：

- lLoginID

- Login handle

- pchDVRIP

- Device IP

- nDVRPort

- Port

- dwUser

- User data, just the same with the above user data you have input.

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：H264_DVR_Cleanup

- CLIENT_API void H264_DVR_Cleanup ();

- API description：Clean up SDK and release occupied resource, calling after all SDK function.

- Parameter：none

- Return：none

- Reference API：H264_DVR_Init

## Get alarm status

typedef bool (__stdcall *fMessCallBack)(long lLoginID, char *pBuf,

unsigned long dwBufLen, long dwUser);

- H264_DVR_API bool H264_DVR_SetDVRMessCallBack(fMessCallBack cbAlarmcallback, unsigned long lUser);

- API description：Set device message callback fuction to get device current state. Callback order does not matter here. SDK default setting is not to callback.You need to call alarm message subscription interface H264_DVR_SetupAlarmChan().It applies to current defined alarm status.Device state is callbacked every second.每态次ENT_Cleanup0000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000

- Parameter：

- cbAlarmcallback

- Message callback function.It is to callback device status (such as alarm status). When it is 0, system disables callback.

- [in] lUser

- user self-defined data

    - Callback function parameters:

- lLoginID

- Return value of H264_DVR_Login

- pBuf

Refer to see SDK_AlarmInfo

- dwBufLen

- pBuf length. Unit is byte.

- dwUser

- User self-defined data

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：H264_DVR_SetupAlarmChan、H264_DVR_CloseAlarmChan

- H264_DVR_API long H264_DVR_SetupAlarmChan(long lLoginID);

- API description：Start listening device message. This function is to set callbacking device message or not. Message is callbacked from H264_DVR_SetDVRMessCallBack.

- Parameter：

- [in]lLoginID

- Return value of H264_DVR_Login

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：H264_DVR_SetDVRMessCallBack，H264_DVR_CloseAlarmChan

- H264_DVR_API bool H264_DVR_CloseAlarmChan(long lLoginID);

- API description：Stop lisening one device

- Parameter：

- [in]lLoginID

- Return value of H264_DVR_Login

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：H264_DVR_SetupAlarmChan

## Device Registration

- H264_DVR_API long H264_DVR_Login (char *sDVRIP, unsigned short wDVRPort, char *sUserName, char *sPassword, LPH264_DVR_DEVICEINFO lpDeviceInfo, int *error);

- API description：Login. When device set the user as reuse(device default user such as admin can be reused.). this account can registed several time.

- Parameter：

- [in] sDVRIP

- device IP

- [in] wDVRPort

- device port

- [in] sUserName

- user name

- [in] sPassword

- password

- [out] lpDeviceInfo

- device property. it is a output parameter.

- [out] error

- (when the function returned successfully, the parameter is null.Please refer to error code.

- Return：Return o if failed. Return device ID if succeeded. Using this value(device handle)all operations after successfuly log in can corresponding to the device.

- Reference API：H264_DVR_Logout

- H264_DVR_API long H264_DVR_LoginEx(char *sDVRIP, unsigned short wDVRPort, char *sUserName, char *sPassword, LPH264_DVR_DEVICEINFO lpDeviceInfo, int nType, int *error);

- API description：Register a user to device extension port. support one user specify device.

- Parameter：

- [in] sDVRIP

- device IP

- [in] wDVRPort

- device port

- [in] sUserName

- user name

- [in] sPassword

- password

- [out] lpDeviceInfo

- device property. it is a output parameter.

- [in] nType

- the type as follows:

enum LoginType

{

LOGIN_TYPE_GUI,			///< Local GUI

LOGIN_TYPE_CONSOLE,		///< Console

LOGIN_TYPE_WEB,			///< WEB

LOGIN_TYPE_SNS,			///< SNS

LOGIN_TYPE_MOBIL,		///< Mobile terminal

LOGIN_TYPE_NETKEYBOARD,	///< Netkeyboard

LOGIN_TYPE_SERVER,		///< Center servers

LOGIN_TYPE_AUTOSEARCH,	///< IP search tool

LOGIN_TYPE_UPGRADE,		///< Upgrade tool

LOGIN_TYPE_MEGAEYE,		///< Megaeye

LOGIN_TYPE_NR,

- };

- [out] error

- (when the function returned successfully, the parameter is null.Please refer to error code.

- Return: Return 0 if failed. Return device ID if succeeded.Using this value(device handle)all operations after successfully logged in can corresponding to the device.

- Reference API: H264_DVR_Logout

- H264_DVR_API long H264_DVR_Logout(long lLoginID)

- API description：Logout user

- Parameter：

- [in]lLoginID

- Return value of H264_DVR_Login

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：H264_DVR_Login

## Real-time Monitor

- H264_DVR_API long H264_DVR_RealPlay(long lLoginID, LPH264_DVR_CLIENTINFO lpClientInfo);

- API description：Start real-time monitor.

- Parameter：

- [in]lLoginID

- Return value of H264_DVR_Login

- [in] lpClientInfo

Client information

- Return：Return real-time monitor handle if succeeded, return 0 if failed.

- Reference API：H264_DVR_StopRealPlay，H264_DVR_SetRealDataCallBack

- H264_DVR_API bool H264_DVR_StopRealPlay(long lRealHandle);

- API description：Stop real-time monitor

- Parameter：

- [in]lRealHandle

- Return value of H264_DVR_RealPlay

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：H264_DVR_RealPlay

- H264_DVR_API bool H264_DVR_SetRealDataCallBack(long lRealHandle,fRealDataCallBack cbRealData, long dwUser);

typedef int(__stdcall *fRealDataCallBack) (long lRealHandle, long dwDataType, unsigned char *pBuffer,long lbufsize,long dwUser);

- API description：Set real-time monitor data callback and provides you with data from the device .When cbRealData is NULL, callback ends.

- Parameter：

- [in]lRealHandle

- Return value of H264_DVR_RealPlay

- cbRealData

- It is a callback function to output the current real-time data from the device.

- [in]dwUser

- User data

            - Callback function parameters:

- lRealHandle

- Return value of H264_DVR_RealPlay

- dwDataType

- 0: original data

- 1: Frame data

- 2: yuv data

- 3: pcm audio data

- pBuffer

- call-back data. Everytime calling back data of different lengths according to the different data types (except type 0). Other data types are based on frames, every time it calls back one frame.

- dwBufSize

- length of callback data.(Unit:byte).

- dwUser

- User self-defined

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：H264_DVR_RealPlay、H264_DVR_StopRealPlay

## Playback And Download

- H264_DVR_API long H264_DVR_FindFile(long lLoginID, H264_DVR_FINDINFO* lpFindInfo, H264_DVR_FILE_DATA *lpFileData, int lMaxCount, int *findcount, int waittime = 2000);

- API description：Search record files

- Parameter：

- [in]lLoginID

- Return value of H264_DVR_Login

- [in]lpFindInfo

- Search condition H264_DVR_FINDINFO

- [out]lpFileData

- Returned record file information.It is a H264_DVR_FILE_DATA

- structure array.

- [in]lMaxCount

- The length of lpFileData（Unit: BYTE，The value shall between 100~200*sizeof(H264_DVR_FILE_DATA)）

- [out]filecount

- Returned file amount; It is an output max parameter. You can only search the video files before buffer is full

- [in]waittime

- Waitting time

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：

- H264_DVR_Login,H264_DVR_PlayBackByName,H264_DVR_StopPlayBack,

- H264_DVR_PlayBackControl, H264_DVR_GetFileByName

typedef void(__stdcall *fDownLoadPosCallBack) (long lPlayHandle, long lTotalSize, long lDownLoadSize, long dwUser)

- H264_DVR_API long H264_DVR_PlayBackByName(long lLoginID, H264_DVR_FILE_DATA *sPlayBackFile, fDownLoadPosCallBack cbDownLoadPos, fRealDataCallBack fDownLoadDataCallBack, long dwDataUser);

- API description：Network playback. Please note, when you login one device, one channel can only play one record at one time, while multi-records of the same channel can’t be opened simutaniously.

- Parameter：

- [in]lLoginID

- Return value of H264_DVR_Login

- [in] sPlayBackFile

- Recorded file information return by H264_DVR_FindFile

- [in] cbDownLoadPos

- Progress call-back function

- [in] fDownLoadDataCallBack

- Video data call-back function

- [in]dwUserData

- User self-defined data

            - CallBack function：

- lPlayHandle

- Return value of H264_DVR_PlayBackByName

- dwTotalSize

- Current total play size, unit is KB.

- dwDownLoadSize

- Played size，unit is KB. When value is -1, it means cureent playback is over.

- dwUser

- User data, just the same with user data in the above.

- Return：Return network playbackID if succeeded, return 0 if failed.

- Reference API：

- H264_DVR_Login,H264_DVR_FindFile,H264_DVR_StopPlayBack,

- H264_DVR_PlayBackControl

- H264_DVR_API bool H264_DVR_StopPlayBack(long lPlayHandle);

- API description：Stop playback

- Parameter：

- [in]lPlayHandle

- Playback handle, Such as the return value of H264_DVR_PlayBackByName

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：H264_DVR_PlayBackByName

- H264_DVR_API long H264_DVR_GetFileByName(long lLoginID,H264_DVR_FILE_DATA *sPlayBackFile,char *sSavedFileName, fDownLoadPosCallBack cbDownLoadPos = NULL, long dwDataUser = NULL );

- API description：Download recorded files via the searched information

- Parameter：

- [in]lLoginID

- The return value of H264_DVR_Login

- [in] sPlayBackFile

- Recorded file information pointer.

- [in]sSavedFileName

- The name of file to save(full path).

- [in]cbDownLoadPos

- Download process calls user self-defined data.For download process callback fucntion, please refer to H264_DVR_GetDownloadPos

- [in]dwUserData

- Download process calls user self-defined data.

            - CallBack function：

- lPlayHandle

- Return value of H264_DVR_PlayBackByName

- dwTotalSize

- Current total play size, unit is KB.

- dwDownLoadSize

- Played size，unit is KB. When value is -1, it means cureent playback is over.

- dwUser

- User data, just the same with user data in the above.

- Return：Return download ID if succeeded. Return 0 if failed.

- Reference API：H264_DVR_StopGetFile、H264_DVR_GetDownloadPos

- H264_DVR_API bool H264_DVR_StopGetFile(long lFileHandle);

- API description：Stop downloading files.

- Parameter：

- [in]lFileHandle

- The return value of H264_DVR_GetFileByName

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：H264_DVR_GetFileByName、H264_DVR_GetDownloadPos

- H264_DVR_API int H264_DVR_GetDownloadPos(long lFileHandle);

- API description：It is to get current downloading place for interfaces that do not need to show real-time download progress. It is similar to the download callback function.

- Parameter：

- [in]lFileHandle

- The return value of H264_DVR_GetFileByName

- Return：position(value between 0 to 100)

- Reference API：H264_DVR_GetFileByName、H264_DVR_StopGetFile

## Playback Control

- H264_DVR_API bool H264_DVR_PlayBackControl(long lPlayHandle, long lControlCode,long lCtrlValue);

- API description：Pause , Resume, Seek network playback

- Parameter：

- [in]lPlayHandle

- Play handle, return by H264_DVR_GetFileByName

- [in] lControlCode

enum SEDK_PlayBackAction

{

SDK_PLAY_BACK_PAUSE,		// Pause

SDK_PLAY_BACK_CONTINUE,		// Resume

SDK_PLAY_BACK_SEEK,			// Seek

- };

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：H264_DVR_PlayBackByName、H264_DVR_StopPlayBack

## PTZ Control

- H264_DVR_API bool H264_DVR_PTZControl(long lLoginID,int nChannelNo, long lPTZCommand, bool bStop = false, long lSpeed = 4)

- API description：PTZ control

- Parameter:

[in]lLoginID

- Return value of H264_DVR_Login

[in] nChannelNo

Channel NO. start with 0

[out] lPTZCommand

Commands, see refer to PTZ_ControlType

[in] bStop

Whether stop or not. It applies to PTZ direction andlens operation. When you operate other functions, input this parameter as FALSE.

[out] lSpeed

Step/Speed. The value ranges from 1 to 8. 8 has the highest control capability(4 by default). dwStep is the preset value when you use preset function.

- Return: Succeeded: TRUE, Fail: FALSE

- Reference API：H264_DVR_Login，H264_DVR_RealPlay

## System Configuration

- H264_DVR_API long  H264_DVR_GetDevConfig(long lLoginID, unsigned long dwCommand, int nChannelNO, char * lpOutBuffer, unsigned long dwOutBufferSize, unsigned long* lpBytesReturned,int waittime = 1000);

- API description: Get device configuration information.

- Parameter:

- [in]lLoginID

- The return value of H264_DVR_Login

[in]dwCommand

Configuration type, Please refer to SDK_CONFIG_TYPE

[in]nChannelNO

Channel number. Set as -1 to configure all channels. The parameter is null if command does not need the channel number.

[out] lpOutBuffer

Receive data buffer pointer, buffer length depend on configuration structure size.

[in]dwOutBufferSize

Receive data buffer lenght(Unit:byte)

[out]lpBytesReturned

The data lenght actually received.

[in]waittime

Waiting time

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：H264_DVR_SetDevConfig

- H264_DVR_API long  H264_DVR_SetDevConfig(long lLoginID, unsigned long dwCommand, int nChannelNO, char * lpInBuffer, unsigned long dwInBufferSize, int waittime = 1000);

- API description: Set device configuration information

- Parameter:

- [in]lLoginID

- The return value of H264_DVR_Login

[in]dwCommand

Configuration type, Please refer to SDK_CONFIG_TYPE

[in]nChannelNO

Channel number. Set as -1 to configure all channels. The parameter is null if command does not need the channel number.

[in]lpInBuffer

Data buffer pointer

[in]dwInBufferSize

Data buffer lenght(unit is byte).

[in]waittime

Waitting time

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：H264_DVR_GetDevConfig

## Log Management

- H264_DVR_API bool H264_DVR_FindDVRLog(long lLoginID, SDK_LogSearchCondition *pFindParam, SDK_LogList *pRetBuffer, long lBufSize, int waittime = 2000);

- API description：Log query

- Parameter：

- [in]lLoginID

- The return value of H264_DVR_Login

- [in] pFindParam

- Search condition, Please refer to SDK_LogSearchCondition

- [in] pRetBuffer

- The return of log information, Please refer to SDK_LogList

- [in] lBufSize

- The return length of log information

- [in]	waittime

- Waiting time

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：None

## Remote Control

- H264_DVR_API bool H264_DVR_ControlDVR(long lLoginID, int type, int waittime = 2000)

- API description：Reboot device and clear log

- Parameter：

- [in]lLoginID

- The return value of H264_DVR_Login

- [in] type

- 0: reboot device，1: clear log

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API：None

## 3.11 Upgrade

typedef void(__stdcall *fUpgradeCallBack) (long lLoginID, long      lUpgradechannel, int nTotalSize, int nSendSize, long dwUser);

- H264_DVR_API long H264_DVR_Upgrade(long lLoginID, char *sFileName, int nType = 0, fUpgradeCallBack cbUpgrade = NULL, long dwUser = 0);

- API description：Upgrade device

- Parameter：

- [in]lLoginID

- The return value of H264_DVR_Login

- [in] sFileName

- The name of upgrade file

- [in] nType

- The type of upgrade file

enum UpgradeTypes

{

UPGRADE_TYPES_SYSTEM,	///< System

UPGRADE_TYPES_NR,

- };

            - CallBack function：

- fUpgradeCallBack

- Callback of upgrade progress, lUpgradechannel is the upgrade handle, return by H264_DVR_Upgrade

- nTotalSize

- Upgrade file lenght,(Unit: BYTE)

- nSendSize

- data length have been upgraded,(unit: BYTE)

- [in]dwUser

- User self-define data

- Return：Succeeded: return upgrade handle, Fail: FALSE

- Reference API:H264_DVR_GetUpgradeState,H264_DVR_CloseUpgradeHandle

- H264_DVR_API long H264_DVR_CloseUpgradeHandle(long lUpgradeHandle);

- API description：stop upgrade

- Parameter：

- [in]lUpgradeID

- The return value of H264_DVR_Upgrade

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API: H264_DVR_Upgrade

- H264_DVR_API int H264_DVR_GetUpgradeState(long lUpgradeHandle)

  - API description：get upgrade status

  - Parameter：

- [in] lUpgradeHandle

- The return value of H264_DVR_Upgrade

- Return：1: Succeeded 2: Upgrading, 3: FALSE

- Reference API: H264_DVR_Upgrade，H264_DVR_CloseUpgradeHand

- H264_DVR_API bool H264_DVR_SearchDevice(char* szBuf, int nBufLen, int* pRetLen,

int nSearchTime);

  - API description：Search device in LAN

  - Parameter：

[in] szBuf

Buffer to receive structure SDK_CONFIG_NET_COMMON,  return the a structure as long as it search a device.

[in] nBufLen

szBuf buffer length

[in] pRetLen

Return the structure total length of SDK_CONFIG_NET_COMMON

[in] nSearchTime

Waiting time

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API:

## Audio Talk

typedef void (__stdcall *pfAudioDataCallBack)(long lVoiceHandle, char *pDataBuf,

long dwBufSize, char byAudioFlag, long dwUser);

- H264_DVR_API long H264_DVR_StartVoiceCom_MR (long lLoginID, pfAudioDataCallBack pVcb,

Long  dwDataUser);

  - API description：Send audio talk request to device.

  - Parameter：

[in] lLoginID

The return value of H264_DVR_Login

[in] pVcb

Self-defined data callback interface.

[in] dwDataUser

Self-defined data. Returned to you via callback function

- Return：>0 audio talk handle, <= FALSE

- Reference API:

H264_DVR_VoiceComSendData, H264_DVR_StopVoiceCom, H264_DVR_SetTalkMode

- H264_DVR_API bool H264_DVR_VoiceComSendData (long lVoiceHandle, char *pSendBuf,

long lBufSize);

  - API description：Send user audio data to device.

  - Parameter：

[in] lVoiceHandle

Return value of H264_DVR_StartVoiceCom_MR

[in] pSendBuf

The audio data to be sent out.

[in] lBufSize

Audio data length to be sent out.(Unit:byte)

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API:

H264_DVR_StartVoiceCom_MR   H264_DVR_StopVoiceCom   H264_DVR_SetTalkMode

- H264_DVR_API bool H264_DVR_StopVoiceCom (long lVoiceHandle);

  - API description：Stop audio talk

  - Parameter：

[in] lVoiceHandle

The return value of H264_DVR_StartVoiceCom_MR

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API:

H264_DVR_StartVoiceCom_MR   H264_DVR_VoiceComSendData   H264_DVR_SetTalkMode

- H264_DVR_API bool H264_DVR_SetTalkMode (long lLoginID, SDK_AudioInFormatConfig* pTalkMode);

  - API description：set audio talk mode

  - Parameter：

    - [in] lLoginID

The return value of H264_DVR_Login

    - [in] pTalkMode

Mode of audio talk ,see refer to SDK_AudioInFormatConfig

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API:

H264_DVR_StartVoiceCom_MR   H264_DVR_VoiceComSendData   H264_DVR_StopVoiceCom

## Record Mode

- H264_DVR_API bool H264_DVR_StartDVRRecord(long lLoginID, int nChannelNo ,long lRecordType);

  - API description：set record mode

  - Parameter：

lLoginID

the return value of H264_DVR_Login

[in] nChannelNo

Channel No., -1 all of channel

[in] lRecordType

Record mode, see refer to SDK_RecordModeTypes

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API:  H264_DVR_StopDVRRecord

- H264_DVR_API bool H264_DVR_StopDVRRecord(long lLoginID, int nChannelNo);

  - API description：stop record

  - Parameter：

[in] lLoginID

The return value of H264_DVR_Login

[in] nChannelNo

Channel No., -1: all of channel

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API:  H264_DVR_StartDVRRecord

## Set System Time

- H264_DVR_API bool H264_DVR_SetSystemDateTime (long lLoginID, SDK_SYSTEM_TIME *pSysTime);

  - API description：set system time

  - Parameter：

[in] lLoginID

The return value of H264_DVR_Login

[in] pSysTime

System time, see refer to SDK_SYSTEM_TIME

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API:  None

## Get Device status

- H264_DVR_API bool H264_DVR_GetDVRWorkState(long lLoginID, SDK_DVR_WORKSTATE *pWorkState);

  - API description：Get device working status

  - Parameter：

[in] lLoginID

The return value of H264_DVR_Login

    - [in] pWorkState

Structure of work staus, see refer to SDK_DVR_WORKSTATE

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API:  None

## Net Keyboard

- H264_DVR_API bool H264_DVR_ClickKey (long lLoginID, SDK_NetKeyBoardData *pKeyBoardData);

  - API description：send net keyboard message

  - Parameter：

[in] lLoginID

The return value of H264_DVR_Login

    - [in] pKeyBoardData

Key value, see refer to SDK_NetKeyBoardData

- Return：Succeeded: TRUE, Fail: FALSE

- Reference API:  None